WAGF: A Governance Framework for LLM-Driven Agent-Based Models of Human-Water Systems

Wenyu Chen1, Second Author1

1Department of Civil and Environmental Engineering, Lehigh University, Bethlehem, PA, USA

Corresponding author: Wen-Yu Chen (wec324@lehigh.edu)

Key Points

- WAGF reduces feasibility hallucination (R_H) and rationality deviation (R_R) relative to ungoverned LLM agents, while preserving high behavioral diversity.

- Effective Behavioral Entropy (EBE) metric separates true decision diversity from hallucination-inflated entropy

- Governance rules transfer across domains: flood adaptation (100 agents, 10 yr) and irrigation (78 agents, 42 yr).

Abstract

Large language models (LLMs) offer a promising path toward cognitively realistic agent-based models (ABMs) for water resources planning, but unconstrained LLM agents can produce state-inconsistent and behaviorally incoherent decisions. We present WAGF (Water Agent Governance Framework), an open-source governance framework that enforces domain-specific physical and institutional constraints while preserving emergent behavioral diversity. WAGF integrates three components: (1) a rule-based validator chain, (2) a tiered cognitive memory system, and (3) a priority context builder. We report two complementary diagnostics: rationality deviation (R_R) and feasibility contradiction (R_H), and quantify diversity with Effective Behavioral Entropy (EBE). In flood adaptation experiments (100 agents, 10 years, three Gemma 3 sizes), governance reduces coherence deviations while maintaining higher effective diversity. We further demonstrate transferability in Colorado River irrigation management (78 districts, 42 years).

Plain Language Summary

Artificial intelligence language models can power virtual agents that make human-like decisions in water management simulations. Without governance, these agents may produce infeasible or behaviorally inconsistent choices. We developed WAGF, a governance framework that checks each decision against identity and physical constraints, then applies coherence checks, before execution. This process improves behavioral rationality while preserving diverse, realistic choices. We demonstrate the approach in two water domains: household flood adaptation and Colorado River irrigation management.

1. Introduction

Agent-based models (ABMs) are a core method for coupled human-water analysis, including flood adaptation and irrigation under scarcity (Aerts et al., 2018; Di Baldassarre et al., 2013; Hung & Yang, 2021). In water resources practice, many ABMs encode behavior through predefined, theory-grounded rules and utility assumptions, which support interpretability and calibration but limit adaptive expressivity (Filatova et al., 2013; Berglund, 2015). Large language models (LLMs) provide a complementary pathway: language-driven agents that produce context-dependent behavioral responses and can provide richer heterogeneity than fixed-rule policies (Park et al., 2023; Gao et al., 2024).

This opportunity introduces a governance challenge. Without explicit controls, LLM agents can produce physically infeasible actions (behavioral hallucination), economically incoherent actions (operationalized in Section 3 as economic hallucination), and state-inconsistent action patterns that are difficult to detect from prompts alone (Ji et al., 2023). In water ABMs, these failures include re-adopting already completed measures, violating legal allocation constraints, or repeatedly selecting extreme actions under incompatible system states. In our experiments, prompt-only steering was insufficient; feasibility checks and audit traces were required to avoid overestimating behavioral diversity from invalid actions.

This Technical Note addresses that gap with three contributions. First, it presents WAGF (Water Agent Governance Framework), an open-source governance framework that sits between LLM reasoning and simulation execution. WAGF integrates a rule-based validator chain, skill-level action contracts and proposal checks, retry-with-feedback control, and structured audit logging. Second, it introduces Effective Behavioral Entropy (EBE), a metric that separates genuine decision diversity from hallucination-inflated diversity. Third, it demonstrates transfer across two configured domains, household flood adaptation (100 agents, 10 years) and Colorado River irrigation management (78 districts, 42 years), showing that governance can enforce physical and institutional feasibility while preserving bounded, human-like behavioral variation.

2. WAGF Architecture

2.1 System Boundary and Broker Pipeline

WAGF is a governance framework between LLM reasoning and simulation state transition. Identity rules and physical rules are enforced as first-pass constraints before thinking-level coherence checks, so infeasible proposals are filtered prior to execution.

Each decision follows a broker pipeline aligned with the architecture components. The Context Builder assembles agent state, environment signals, and memory cues. The Model Adapter and Parser converts LLM output into a normalized skill proposal using layered fallbacks (strict JSON, enclosure delimiters, regex extraction, numeric fallback). The Skill Registry resolves the proposed skill and checks declared preconditions/postconditions. The Validator then applies prioritized rules and returns approve, warn, or block outcomes. Blocked proposals are returned with rule-level feedback for bounded retry.

Only validated skills are handed to Action Execution. In parallel, the Auditor records proposal content, validation outcomes, and execution status as structured traces for reproducibility and diagnosis. Observation outcomes are then routed to memory retrieval and reflection for subsequent turns, closing the agent-environment loop without bypassing governance checks.

This sequence translates LLM proposals into executable actions under explicit constraints and full traceability.

2.2 Cross-Cutting Governance Components

WAGF applies three cross-cutting governance components across this pipeline. Rules are prioritized (identity, thinking, warning), so physical impossibilities are rejected before softer coherence checks. Cognitive memory supports both a sliding-window baseline and a human-centric mechanism with salience-weighted encoding and stochastic consolidation/decay. Priority context enforces tiered prompt assembly (must include, should include, nice to have) to preserve safety-critical state information under token constraints. Together, these components improve behavioral plausibility while preserving bounded heterogeneity.

2.3 Domain Instantiation

WAGF is domain-agnostic at the engine level and domain-specific at the configuration level. The same broker runtime is reused across household flood adaptation and Colorado River irrigation management; adaptation is expressed through YAML skill definitions, validator rules, and prompt templates rather than core-code branching. In the flood case, skills include elevation, insurance purchase, and relocation under a Protection Motivation Theory scaffold. In the irrigation case, skills include demand adjustment, efficiency adoption, and acreage reduction under a dual-appraisal (Water Scarcity Assessment and Adaptive Capacity Assessment) scaffold. This configuration-first pattern supports transfer to additional water-system settings with a common governance contract.

Table 1. Cross-model governance effectiveness in flood adaptation (100 agents, 10 years). Ungov. = Group A (raw LLM), WAGF = governed configurations. Main table columns emphasize R_R and diversity retention (H_norm, EHE). R_H is retained as a strict feasibility safety diagnostic in SI.

| Model | Ungov. RH(%) | WAGF RH(%) | Reduction (%) | EBE Gain (%) |
| --- | --- | --- | --- | --- |
| Gemma3-4B | 20.84 | 2.80 | 86.6 | 48.7 |
| Gemma3-12B | 9.71 | 0.00 | 100.0 | 11.1 |
| Gemma3-27B | 8.70 | 0.00 | 100.0 | 7.9 |
| Ministral-3B | 13.82 | 0.13 | 99.1 | 70.4 |
| Ministral-8B | 11.99 | 0.00 | 100.0 | -5.0 |
| Ministral-14B | 11.61 | 0.00 | 100.0 | 67.8 |

3. Metrics

We distinguish two validation failure types. Feasibility hallucination occurs when a proposed skill a_t violates identity constraints under agent state s_(t-1) and active institutional conditions. Let A_id(s_(t-1)) denote the feasible skill set under identity checks; a proposal is a feasibility hallucination when a_t not in A_id(s_(t-1)). Thinking-rule violations are treated separately as rationality deviations (coherence failures) rather than hallucinations.

This separation avoids conflating physically or institutionally infeasible behavior with boundedly rational but feasible behavior.

R_H = n_id / n_active          (1)

R_R = n_think / n_active          (2)

R_H is the feasibility hallucination rate (decision-level identity/feasibility violations over active decisions), while R_R is the rationality-deviation rate (decision-level thinking-rule coherence violations over active decisions). These numerators are counted at decision level and are not equal to retry workload metrics (retry_rows, retry_sum). We define Effective Behavioral Entropy (EBE) as diversity adjusted by feasibility burden:

Here k is the number of available skills and p_i is the observed frequency of skill i. For corrected entropy reporting, hallucinated proposals are remapped to the domain fallback skill (DoNothing in flood, maintain_demand in irrigation). We report R_H, R_R, raw entropy, corrected entropy, and EBE jointly for transparent comparison (Jost, 2006; Ji et al., 2023).

4. Case Study 1: Flood Adaptation

4.1 Experimental Design

We simulate 100 household agents making annual flood adaptation decisions over 10 years in a flood-prone community. Flood events occur in years 3, 4, and 9, with a baseline annual flood probability of 0.20. Agents choose among five actions: elevate their home, purchase insurance, relocate, both (elevate + insure), or do nothing.

We compare three governance configurations using Gemma 3 4B (Gemma Team, 2024) as the primary LLM: Group A (ungoverned; raw LLM output executed directly), Group B (WAGF governance with sliding-window memory), and Group C (WAGF governance with human-centric memory and priority context schema). All groups use identical initial agent profiles, flood sequences, and random seeds to isolate the effect of governance.

4.2 Feasibility Hallucination and Rationality Diagnostics

We separate validation outcomes into feasibility hallucination and rationality deviation. Feasibility hallucination is evaluated with identity-rule checks against prior state: proposing elevation for an already elevated household is counted as a feasibility violation (R_H channel). Rationality deviation is evaluated with thinking-rule coherence checks between appraisal and selected action (R_R channel). Both channels are computed at decision level over active decisions, while retry counters are reported separately as governance workload.

Under the current strict feasibility definition (identity/precondition contradictions), R_H remains near zero across most runs. The dominant residual channel is coherence deviation (R_R), and governance (Groups B/C) substantially reduces this channel relative to ungoverned baselines.

4.3 Results

Figure 2 presents the key results. The central performance pattern is lower coherence deviation (R_R) under governance with retained effective diversity (EHE). Raw entropy alone can be misleading when feasibility and coherence channels are not reported jointly; therefore we report R_R, R_H, H_norm, and EHE together.

The most striking behavioral difference is relocation. Group A produces zero cumulative relocations: agents repeatedly choose "do nothing" or hallucinate impossible actions, never reaching the psychological threshold for relocation. Group B achieves 32% cumulative relocation and Group C reaches 37%, demonstrating that governance enables rather than constrains meaningful behavioral diversity. The year-9 flood event is particularly informative: Group C adds 12 new relocations (memory of years 3-4 floods amplifies the response), while Group B adds only 1 (window memory has already forgotten early trauma).

[FIGURE 2: Flood Results ??(a) Raw vs corrected entropy with EBE, (b) Cumulative relocation curves]

Table 1 (continued). Detailed model-level values are provided in the companion table sources (`Table1_main_metrics_v8.csv` for main results and SI tables for feasibility diagnostics).

| Metric | Group A (Ungoverned) | Group B (WAGF+Window) | Group C (WAGF+Human.) |
| --- | --- | --- | --- |
| Hallucination Rate (RH) | 20.84% | 0.58% | 2.80% |
| Total hallucinations | 208 | 5 | 24 |
| Raw Hnorm | 0.664 | 0.790 | 0.804 |
| Corrected Hnorm | - | - | - |
| EBE | 0.526 | 0.785 | 0.782 |
| Relocated (Year 10) | 0 / 100 | 32 / 100 | 37 / 100 |
| Active agents (Year 10) | 100 | 68 | 63 |

Pairwise tests (Mann-Whitney U) are interpreted under the updated metric split: R_R as the primary behavioral-rationalization channel, with R_H as a strict secondary safety diagnostic.

5. Case Study 2: Colorado River Irrigation

5.1 Setup

To evaluate transferability, we configure WAGF in a Colorado River irrigation-demand setting while keeping the core governance runtime unchanged from the flood case. The domain adaptation is implemented through configuration-level substitutions, including skill definitions, validator rules, appraisal constructs, and persona templates. In this domain, agents use a dual-appraisal structure, Water Scarcity Assessment (WSA) and Adaptive Capacity Assessment (ACA), which is better aligned with chronic, continuous decision pressure than acute hazard-response settings. This design choice preserves theoretical alignment while maintaining a consistent governance workflow across domains.

Governance is applied in ordered layers: identity and physical feasibility constraints first, followed by construct-conditioned coherence checks, retry-with-feedback, and auditable execution outcomes. Figure 3 shows that this reconfigured setup yields plausible long-horizon system behavior under institutional and hydrologic constraints, while preserving bounded behavioral heterogeneity at the agent level. The irrigation case is therefore presented as transferability evidence: the same framework can be reused across water domains without redesigning core architecture.

5.2 Results

Figure 3 presents the transferability demonstration in the irrigation domain. Panel (a) shows system-level demand trajectories against the reference envelope across the simulation horizon. Panel (b) summarizes governance-mediated execution outcomes over time. The emphasis is not on introducing additional performance metrics, but on demonstrating that the same governance framework remains operational, interpretable, and behaviorally plausible after domain reconfiguration.

Together, the two panels provide visual evidence that WAGF can be transferred from flood adaptation to irrigation-demand management using configuration-level changes, while retaining feasible action filtering and interpretable decision dynamics under chronic scarcity.

The governance process visibly narrows the executable action space while retaining interpretable differences across behavioral clusters, indicating bounded heterogeneity rather than homogeneous lock-step behavior. This pattern is consistent with the chapter goal: transferability with governance-legible dynamics, not optimization-centric performance reporting.

[FIGURE 3: Irrigation Results - (a) Demand trajectories, (b) Governance outcomes]

Figure 3. Irrigation transferability demonstration under WAGF in the Colorado River setting. The panel shows system-level demand trajectories against the reference envelope and governance-mediated execution outcomes over time. Results indicate that a common governance framework can be reconfigured across domains while preserving feasible and behaviorally interpretable dynamics.

[Figure 3. Irrigation case study (78 agents, 42 years, gemma3:4b). (a) Annual aggregate demand vs CRSS baseline (dashed) with ±10% corridor (shaded). (b) Governance intervention outcomes: persistent ~60% intervention rate reflects structural bounded rationality requiring continuous governance constraint.]

6. Discussion

EBE provides a diagnostic that raw entropy alone cannot, but its interpretation is specific: it adjusts diversity by feasibility hallucination burden (R_H). In Group A, raw H_norm averages 0.664, while EBE is 0.526 after feasibility correction, consistent with substantial infeasible proposal mass. This indicates that diversity estimates from LLM-ABM outputs can be inflated when feasibility is not accounted for. For this reason, we recommend reporting raw entropy, corrected entropy, and R_H together rather than raw entropy alone (Jost, 2006).

Cross-model testing across Gemma 3 model sizes (4B, 12B, 27B) suggests that governance mitigates degenerate action concentration in ungoverned settings. In our runs, Gemma 3 12B under Group A shows high infeasible/degenerate behavior and near-zero EBE, whereas governed settings retain non-zero effective diversity. Under the Section 3 taxonomy, EBE tracks feasibility-adjusted diversity (R_H channel), while rationality/coherence deviations are tracked separately through R_R audit traces; these two diagnostics should be interpreted jointly rather than interchangeably (Gemma Team, 2024).

A structural distinction between reinforcement-learning agents and LLM agents informs governance design. In the FQL baseline (Hung and Yang, 2021), agents update Q-values through a reward function that includes a regret penalty for over-demand; this endogenous feedback enables self-regulation without external constraints. LLM agents, by contrast, receive individual-level feedback through episodic memory (e.g., “I requested 115,000 AF but received only 95,000 AF”) and reflective summaries, but do not observe basin-level externalities: no agent is informed that collective demand exceeded available supply or that its increase contributed to another agent’s curtailment. This asymmetry — individual consequence visibility without commons-level attribution — means that even with multi-year memory, agents cannot learn to self-regulate aggregate demand. The governance layer compensates for this gap by enforcing institutional constraints (demand corridor, shortage-tier curtailment) that are functionally equivalent to the reward-based convergence mechanism in FQL. This design mirrors real-world water governance, where administrative rules (compacts, annual allocation limits) constrain behavior rather than relying on individual actors’ experiential learning (Schlager and Ostrom, 1992; Pulwarty and Melis, 2001).

Several limitations remain. First, primary results are based on single-seed runs; flood experiments use N=100 agents for within-run contrast, and irrigation results (78 agents, seed 42) have not been evaluated across alternative random seeds. Second, LLM sampling sensitivity (temperature, top-p, top-k) is not systematically evaluated. Third, validator rules are expert-authored; automated rule induction remains future work. Fourth, R_R is currently used as an audit diagnostic and is not yet summarized with the same inferential depth as R_H/EBE across all case studies. Finally, warning-level governance is intentionally non-blocking, so behaviorally questionable yet technically feasible decisions can still pass execution. Sixth, the reflection mechanism provides agents with individual action-outcome feedback but not basin-level externalities; whether richer system-level feedback could reduce governance dependence with more capable LLMs remains untested.

7. Conclusions

We present WAGF as a governance framework for LLM-driven agent-based modeling of human-water systems

In the flood case, strict-feasibility R_H is near zero in current runs, while coherence deviations (R_R) remain materially higher in ungoverned settings

Second, governance reduces both feasibility hallucination (R_H) and irrational behavior (R_R) while maintaining high behavioral diversity. Governed groups show higher EBE than the ungoverned baseline and substantially higher cumulative relocation in flood adaptation, indicating that governance removes infeasible/irrational mass without collapsing the action repertoire.

Third, the architecture transfers across flood and irrigation without broker-level code changes: the same runtime is instantiated through domain-specific YAML artifacts (skills, validators, constructs, personas). This supports reuse of governance infrastructure across socio-hydrological ABM settings (Giuliani et al., 2022; Castelletti et al., 2010).

Future work should extend multi-agent interaction governance, develop data-informed rule induction, and provide fuller inferential summaries for R_R alongside R_H/EBE across domains. Additionally, injecting system-level feedback into agent reflection — such as basin-wide supply-demand balance or collective action outcomes — could test whether larger LLMs develop commons awareness that reduces dependence on hard governance constraints, bridging the gap between reward-based self-regulation and institutional rule enforcement. WAGF, metric definitions, and experiment code are released open-source at [GitHub URL].

Data Availability Statement

Simulation logs, agent configurations, and analysis scripts are archived at [Zenodo DOI] and available at [GitHub URL] under the MIT License. The CRSS precipitation projections are from the U.S. Bureau of Reclamation (USBR, 2012). All raw LLM traces (JSONL format) and governance audit logs are included in the archive to support independent verification.

Conflict of Interest

The authors declare no conflicts of interest relevant to this study.

Author Contributions

Wen-Yu Chen: Conceptualization, Methodology, Software, Validation, Formal analysis, Investigation, Writing ??Original Draft, Visualization. [Second Author]: Supervision, Writing ??Review & Editing, Funding acquisition.

Acknowledgments

This work was supported by [funding source].

References

Aerts, J. C. J. H., et al. (2018). Integrating human behaviour dynamics into flood disaster risk assessment. Nature Climate Change, 8(3), 193??99.

Bai, Y., et al. (2022). Constitutional AI: Harmlessness from AI feedback. arXiv:2212.08073.

Berglund, E. Z. (2015). Using agent-based modeling for water resources planning and management. J. Water Resour. Plan. Manag., 141(11), 04015025.

Boiko, D. A., et al. (2023). Emergent autonomous scientific research capabilities of large language models. Nature, 624, 570??78.

Bubeck, P., et al. (2012). A review of risk perceptions and other factors that influence flood mitigation behavior. Risk Analysis, 32(9), 1481??495.

Castelletti, A., et al. (2010). Tree-based reinforcement learning for optimal water reservoir operation. Water Resour. Res., 46(9), W09507.

Di Baldassarre, G., et al. (2013). Socio-hydrology: Conceptualising human-flood interactions. Hydrol. Earth Syst. Sci., 17(8), 3295??303.

Filatova, T., et al. (2013). Spatial agent-based models for socio-ecological systems. Environ. Model. Softw., 45, 1??.

Gao, C., et al. (2024). Large language models empowered agent-based modeling and simulation: A survey. Humanit. Soc. Sci. Commun., 11, 1498.

Gemma Team (2024). Gemma: Open models based on Gemini research and technology. arXiv:2403.08295.

Giuliani, M., et al. (2016). Is robustness really robust? Climatic Change, 135(3), 409??24.

Giuliani, M., et al. (2022). Is it worth using AI to solve the water crisis? Water Resour. Res., 58(4), e2021WR031219.

Guo, D., et al. (2025). DeepSeek-R1: Incentivizing reasoning capability in LLMs via reinforcement learning. arXiv:2501.12948.

Hadjimichael, A., et al. (2020). Defining robustness for diverse stakeholder interests in institutionally complex river basins. Earth's Future, 8(7), e2020EF001503.

Herman, J. D., et al. (2015). How should robustness be defined for water systems planning under change? J. Water Resour. Plan. Manag., 141(10), 04015012.

Hung, C.-L., & Yang, Y. C. E. (2021). An agent-based modeling approach for water resources management under uncertainty. Water Resour. Res., 57(10), e2021WR030519.

Ji, Z., et al. (2023). Survey of hallucination in natural language generation. ACM Comput. Surv., 55(12), 1??8.

Jost, L. (2006). Entropy and diversity. Oikos, 113(2), 363??75.

Kanta, L., & Zechman, E. (2014). Complex adaptive systems framework for urban water resources. J. Water Resour. Plan. Manag., 140(1), 75??5.

Ollama (2024). Ollama: Run large language models locally. https://ollama.com

Ouyang, L., et al. (2022). Training language models to follow instructions with human feedback. NeurIPS, 35, 27730??7744.

Park, J. S., et al. (2023). Generative agents: Interactive simulacra of human behavior. ACM UIST, 1??2.

Rao, A. S., & Georgeff, M. P. (1995). BDI agents: From theory to practice. Proc. 1st Int. Conf. Multi-Agent Syst., 312??19.

Rogers, R. W. (1975). A protection motivation theory of fear appeals and attitude change. The Journal of Psychology, 91(1), 93??14.

Rogers, R. W. (1983). Cognitive and psychological processes in fear appeals and attitude change. In J. Cacioppo & R. Petty (Eds.), Social Psychophysiology (pp. 153??76). Guilford Press.

Shannon, C. E. (1948). A mathematical theory of communication. Bell Syst. Tech. J., 27(3), 379??23.

Shumailov, I., et al. (2024). AI models collapse when trained on recursively generated data. Nature, 631, 755??59.

Sivapalan, M., et al. (2012). Socio-hydrology: A new science of people and water. Hydrol. Process., 26(8), 1270??276.

Touvron, H., et al. (2023). Llama 2: Open foundation and fine-tuned chat models. arXiv:2307.09288.

U.S. Bureau of Reclamation (2012). Colorado River Simulation System (CRSS). Washington, DC.

Wei, J., et al. (2022). Chain-of-thought prompting elicits reasoning in large language models. NeurIPS, 35, 24824??4837.

Yang, Y. C. E., et al. (2009). A decentralized optimization algorithm for multiagent system-based watershed management. Water Resour. Res., 45(8), W08430.